MIDTERM PHASE 0 - HUMAN DETECTOR AND TRACKER

#### 1. Overview

This document outlines the plan for the design and development of a new Human Obstacle Detector and Tracker module for Acme Robotics. The purpose of this module is to identify the 3D location of one or more humans in a robot's environment using a single monocular video camera.

This component is critical for safe human-robot interaction and navigation. It will provide downstream components, such as a motion planner, with a list of detected human obstacles and their positions relative to the robot. This allows the robot to implement behaviors like stopping, slowing down, or navigating around people.

The module will interface with the robot's camera system by accepting raw video frames. It will output a structured data list of DetectedObjects, where each object contains a unique tracking ID and its (x, y, z) position in the robot's reference frame.

Core Assumptions:

- The module receives a continuous video stream from a single, calibrated monocular camera or a clear well lit image.

- The robot's control system can provide the camera's mounting parameters (height from the ground, pitch angle).

- Humans are walking on a flat, continuous ground plane.

- Occlusion handling (i.e., tracking a person after they go behind another object) is not required for this phase.

#### 2. Reference Materials

- Software Engineering: The Current Practice by Václav Rajlich (provided textbook)

- Project Assignment Brief (Acme Robotics Mid-Term Project)

- Google C++ Style Guide

- OpenCV Library Documentation

#### 3. Definitions and Acronyms

- AIP: Agile Iterative Process

- CI: Continuous Integration

- TDD: Test-Driven Development

- UML: Unified Modeling Language

- DNN: Deep Neural Network

- Monocular Camera: A single camera, which provides 2D images without inherent depth information.

- Robot Reference Frame (Pose): A 3D coordinate system centered on the robot.

#### 4. Process

We will follow the Agile Iterative Process (AIP) as described in Chapter 13 of the textbook , combined with a Test-Driven Development (TDD) methodology.

- Phase 0 (Week 1): Planning & Design.

- Phase 1 (Week 2): Sprints 1 & 2.

  - Sprint 1: Implement core classes, camera interface, and 2D detection (TDD).

  - Sprint 2: Implement 2D tracker and 3D location estimator (TDD).

- Phase 2 (Week 3): Sprint 3 & Final Delivery.

  - Sprint 3: Integration, system testing, documentation, and code coverage push (to 90%+).

Our development will follow the TDD cycle:

- Write a failing unit test for a new feature (Red).

- Write the minimal code to make the test pass (Green).

- Refactor the code while ensuring all tests still pass (Refactor).

Quality will be ensured via GitHub CI running cppcheck (static analysis), Google Test (unit tests), and CodeCov (coverage monitoring) on every commit.

#### 5. Organization

We are a two-person team. We will execute Pair Programming for all development tasks.

- Driver: The developer writing the code.

- Navigator: The developer reviewing the code in real-time, considering strategic design, and identifying potential bugs or test cases.

- Rotation: We will switch roles for every Different Phase for the assignment.

- Documentation: Our pair programming sessions will be documented in our Github repository.

#### 6. Technologies

- Language: C++17

- Build System: CMake

- Testing: Google Test (gtest)

- Static Analysis: cppcheck

- Documentation: Doxygen

- CI/CD: GitHub Actions

- Coverage: CodeCov

- External Libraries:

  - OpenCV : Used for all image processing, video I/O, and for loading and running the pre-trained DNN model for 2D detection.

  - Eigen : A high-performance C++ template library for linear algebra, which will be used for the 3D ground-plane projection calculations.

#### 7. Management

- Task Management: We will use GitHub Issues to manage our product backlog and sprint tasks.

- Version Control: We will use a main branch for stable releases and feature branches for all new development. All feature branches will require a passing CI build and a review from the other team member before merging into main.

- Risks & Mitigations:

  - Risk: Inaccurate 3D location from monocular data.

    - Mitigation: This is the project's primary technical challenge. We will mitigate it by clearly defining our assumption (flat ground plane) and implementing a well-documented geometric projection algorithm. Accuracy will be dependent on precise camera calibration, which we will simulate in our unit tests.

  - Risk: Real-time performance of the DNN model.

    - Mitigation: We will select a lightweight, high-performance model (YOLO v8) designed for edge devices.

  - Risk: TDD for computer vision is difficult.

    - Mitigation: We will test known inputs. For example, we will save a sample image with known human locations and write a test asserting that our IDetector finds them. We will test the ILocationEstimator by feeding it exact 2D pixel coordinates and asserting the correct 3D output coordinates.

#### 8. Cost

As this is a 3-week academic project, the "cost" is defined by development time.

- Team: 2 developers.

- Project Duration: 3 weeks.

- Total Effort (Estimate): 3 weeks * 2 developers * 20 hours/week = 120 person-hours.

- Software Cost: $0. All development tools and libraries (C++17, CMake, GTest, OpenCV) are free and open-source.